PROJECT REPORT

Wireless Gesture-Controlled Robotic Arm using Flex Sensors

Aim:

To design and implement a wireless gesture-controlled robotic arm using flex sensors and ESP32 microcontrollers.

Apparatus / Components Required:

- ESP32 (2 units)

- Flex Sensors

- Servo Motors

- PCA9685 PWM Driver Module

- 18650 Li-ion Batteries

- Buck Converter (5V regulation)

- Breadboard / Zero PCB

- Connecting Wires

Theory (Detailed Explanation):

This project is based on the concept of gesture-based human-machine interaction, where human hand movements are used to control a robotic system.

1. Flex Sensor

A flex sensor is a variable resistor:

- Straight → low resistance

- Bent → high resistance

This change in resistance is converted into a voltage signal using a voltage divider circuit. The ESP32 reads this voltage using its ADC and determines how much the finger is bent.

2. ESP32 Microcontroller

ESP32 acts as the control unit:

- Reads analog sensor data

- Sends/receives data wirelessly

- Controls output devices

It has built-in Wi-Fi and supports ESP-NOW protocol.

3. ESP-NOW Communication

ESP-NOW is a wireless protocol:

- Direct device-to-device communication

- No router required

- Low latency

Used to send sensor data from transmitter to receiver ESP32.

4. I2C Protocol

I2C is used to communicate between ESP32 and PCA9685:

- SDA → Data line

- SCL → Clock line

- Only 2 wires needed

5. PCA9685 PWM Driver

- Controls up to 16 servos

- Generates PWM signals

- Reduces load on ESP32

6. Servo Motors

- Rotate based on PWM signal

- Used in robotic arm joints

- Replicate finger movement

7. System Architecture Design (Important)

The system is intentionally designed as:

- Transmitter → Only reads and sends data

- Receiver → Processes and controls actuators

Block Diagram:

Working Principle (Step-by-Step):

- Flex sensors detect finger bending

- Resistance changes → voltage changes

- ESP32 reads voltage via ADC

- Transmitter sends raw data using ESP-NOW

- Receiver ESP32 receives data

- Maps values to servo angles

- Sends commands to PCA9685 via I2C

- PCA9685 generates PWM signals

- Servo motors move accordingly

Circuit Description:

🔹 Flex Sensor Circuit

- Connected as voltage divider

- Output connected to ESP32 ADC pin

🔹 ESP32 to PCA9685

- SDA → Data

- SCL → Clock

🔹 PCA9685 to Servo

- PWM output to servo signal pin

🔹 Power System

- 18650 batteries used

- Buck converter regulates to 5V

- Servos require stable current supply

Procedure:

- Connect flex sensors to ESP32 ADC pins

- Write code to read analog values

- Implement ESP-NOW communication

- Interface PCA9685 with receiver ESP32

- Connect servos to PCA9685

- Power system using battery and buck converter

- Test gesture-based control

Code Implementation:

The firmware is divided into two modules:

1. Transmitter Code (Glove Unit)

- Reads analog values from flex sensors

- Sends raw data using ESP-NOW

- No mapping done (keeps design simple)

View Transmitter Code (ESP32):

2. Receiver Code (Robotic Arm Unit)

- Receives data via ESP-NOW

- Maps values to servo angles

- Communicates with PCA9685 using I2C

- Controls servo motors

View Receiver Code (ESP32 + PCA9685):

Programming Details:

- Language: Embedded C (Arduino Framework)

- IDE: Arduino IDE

- Protocols:

  - ESP-NOW (Wireless)

  - I2C (Servo control)

Observations:

- Real-time gesture control achieved

- Wireless communication worked smoothly

- Servo response matched finger movement

- Minor jitter observed in servo movement

Result:

The wireless gesture-controlled robotic arm was successfully implemented and demonstrated. The system worked effectively at the prototype stage.

Challenges Faced:

🔸 Servo Jitter

- Caused by unstable power supply

- Breadboard connections introduced noise

🔸 Flex Sensor Damage

- Hot glue damaged sensing layer

- Reduced sensor performance

🔸 Analog Noise

- Slight fluctuations in readings

Precautions:

- Avoid applying heat on flex sensors

- Use proper mounting methods (tape/clamps)

- Ensure stable power supply

- Avoid loose connections

Improvements:

- Use PCB instead of breadboard

- Add capacitors for filtering

- Improve sensor mounting

Applications:

- Prosthetic hand systems

- Robotics control

- Industrial automation

- Human-machine interfaces

Conclusion:

The project successfully demonstrates a wireless gesture-controlled robotic system using embedded technologies. It highlights the importance of communication protocols, power stability, and mechanical design in real-world applications.